macOS cups Printing Service

(Only support USB & LAN printer)

Run macOS device

Enter installation content, operate sudo ./install.sh

Run Safari brower, and visit http://localhost:631 or http://127.0.0.1:631 enter into Cups printing service. (Notice: If can not open Web page, please run the command cupsctl Webinterface=yes, start Web page, and reflash page.)

Add printer (If need password, please input it)

As picture shows to choose AppSocket

Input IP address

Input printer name, like office printer

Select file 80mmSeries.ppd

Click “Add Printer”, choose page length, set it as default printer

Print test page

Self-test page printing successfully! Driver installation finished.